Manual for WowFunctionTest and WowFunctionTestServer

Outline

WowFunctionTest and WowFunctionTestServer application are test tools for debugging wowl features.

Path to WowFunctionTest directory is:

Wireless/Samples/Sources/Applications/WlanTest/WowFunctionTest

Path to WowFunctionTestServer directory is:

Wireless/Samples/Sources/Applications/WlanTest/WowFunctionTestServer

Preparation

Before using the test tools, you should do the followings:

- Run DevMenu. DevMenu automatically runs when you connect TargetManager to SDEV/EDEV.

Figure 1 Top menu of DevMenu

- Select “Network” from the left side bar:

- Check the box of “Testing mode enabled”

- Install am.withoutPcieWait.nsp I provided on MyCase #00301831. The nsp file is included in ReleaseProcs0314.zip.

- Restart SDEV/EDEV.

How to use

- Run WowFunctionTestServer.

WowFunctionTestServer, which is a windows application is a server application for wowl test. You should run it before running WowFunctionTest, which is client application.

Figure 2 WowFunctionTestServer

WowFunctionTestServer will open a socket on port 45678. If you want to change the port number, please edit the following code:

WowFunctionTestServer/Form1.cs, line 34

private void MainForm_Load(object sender, EventArgs e)

{

m_Server = new Server(45678);    // <---- Change this number

m_Server.Connected += ClientConnected;

m_Server.Run();

}

- Run WowFunctionTest

  - You can run WowFunctionTest by loading WowFunctionTest.nsp with TargetManager. This application will open htcs port, whose name is “wowtest”.

  - Open a terminal window and connect to “wowtest” port.

The message “Hi!” will be displayed.

  - Type “help”, then the usage will be displayed.

- Connect SDEV/EDEV to an AP

You can connect SDEV/EDEV to an AP by using join command.

e.g.) join wowl-feature-test wpa2 012345678

When you succeed in the connection, ip address will be displayed.

- Establish tcp connection

You can establish tcp connection between WowFunctionTest and WowFunctionTestServer by using socket command:

e.g.) socket 192.168.11.4

The connection information will be displayed on the server application window:

The server application will start to send tcp keep alive packet automatically.

- Send tcp data from the server to the client

Press “send data” button of the server application, then a tcp data will be sent to the client. When the client receives it, the data will be dumped on the terminal:

- Set tcp session parameters to WDM

After receiving tcp data, you can use “info” command to set tcp session information.

e.g.) info 1

Then, tcp session information will be displayed on the terminal window.

- Press power button of SDEV/EDEV, then the system goes to wowl sleep mode.

NOTE : Before that, you should detach the joy-cons from the rail of SDEV/EDEV. Otherwise, the system will go to half-awake state automatically in order to prepare charging joy-cons, and after finishing the preparation the system will go to sleep state. But at this time, WDM will disconnect from the AP automatically. So, please detach the joy-cons to make the system enter wowl sleep state.

- The client responds to TCP KeepAlive packet during wowl sleep state. You can see that via sniffer:

- Wake SDEV/EDEV up.

Press “send data” button of the server application, then SDEV/EDEV will wake up due to receiving tcp data.

- Resume the client application.

The client application itself is still sleeping. So, you need to resume it.

  - Press any buttons other than home button of the joy-con, then the joy-con connects to SDEV/EDEV. Then, press home button of the joy-con. The client application will be resumed. But “wowtest” entry will not be displayed on TargetManager window. Nothing seems to be changed. So, it is hard to know if the application is really resumed. You can confirm it by typing “help” on the terminal. If the application is resumed, the usage will be displayed.

- Close socket.

  - Use “close” command to close the socket.
e.g.) socket 1

  - The client application will show the result on the terminal window. And the check mark will disappear from the server application window:

- Disconnect from the AP

  - Use “disconnect” command to disconnect from the AP.

- Exit the client application

  - Use “exit” command.

Additional commands

set_wake

set_wake <hex>

You can set wake reason directly by using this command

e.g.) set_wake 0x10041C

“0x10041C” means (WL_WOWL_DIS | WL_WOWL_RETR | WL_WOWL_BCN | WL_WOWL_GTK_FAILURE | WL_WOWL_TCPKEEP_DATA)

NOTE: Normally, wlan process does not set net pattern. So, if you set WL_WOWL_NET by this command, bwl_wowl() will return error code -2 and wlan process will crash. Wlan process is implemented to crash when bwl_wowl() returns error code.

get_wake

get_wake

You can get wake reason directly by using this command instead of “wake_reason” command. The wake reason is displayed as hex.

wowl_feature

wowl_feature [<arp|tcpka>] …

You can enable/disable ARP offload feature and TCP KA feature by using this command.

e.g.) wowl_feature arp tcpka

In the above case, ARP offload feature and TCP KA feature will be enabled. The feature that you don’t set here will be disabled. Currently, we have just 2 features (ARP offload and TCP KA feature).

e.g.) wowl_feature tcpka

“arp” feature is not included in the above case. This will disable ARP offload feature and enable TCP KA feature.

If you don’t use this command at all, wlan process will enable default features (ARP offload and TCP KA).

Change log

- Rev 001

  - Initial

- Rev 002

  - Add description of the following command:

    - set_wake

    - get_wake

    - wowl_feature