- EBO Server Debug Guide - DRAFT Updated July 31, 2026 Prepared by: Fred Koopmans Bytemobile, Inc. 2025 Stierlin Court Mountain View, CA 94043, USA Tel: 1-650-641-7700 Fax: 1-650-641-7701

# Revision Control

| Version # | Date | Author | Changes |
| --- | --- | --- | --- |
| DRAFT | 09/22/2006 | Fred Koopmans | - First external draft |

Revision Control	2

1	Web Proxy Log Files	4

1.1	Access Log File	4

1.1.1	Fields in the Access Log File	4

1.1.2	Cache Codes	7

1.1.3	HTTP Status Codes	8

1.2	Information Log File	8

1.2.1	Fields in the Information Log File	8

1.2.2	Controlling Log Messages	9

1.3	Fatal Log File	9

1.4	Log File Rotation	9

2	Web Proxy Configuration Options	10

# Web Proxy Log Files

There are three separate log files associated with the Web Proxy service, and are listed below.

- The Access Log file records HTTP request and response transaction information. The default location of this file is /opt/bmi/var/log/webgrp/web_access.log.

- The Information Log file records informational, warning, and error messages generated by the Web Proxy service. The default location of this file is /opt/bmi/var/log/webgrp/web_info.log.

- The Fatal Log file contains debug messages generated by the Web Proxy service when an unexpected and fatal condition is encountered that would eventually force the service to terminate. This log file is primarily used as a debugging tool to isolate service affecting conditions. The default location of this file is /opt/bmi/var/log/webgrp/fatal.log.

## Access Log File

The Access Log file records HTTP request-response transactions. There is a single Access Log file irrespective of the number of Web Proxy processes. Each HTTP request-response transaction is entered into a separate line in the Access Log file. An example of an entry in the Access Log file is shown below.

Fri Apr 18 16:30:40 2003 p933   612       6000/001     10.0.103.52:3982(23464):0 =>    64.236.24.12:80:0  803ms    56ms      53369 ->     13168   75%          MISS/200 G--H--M-U-P—X---  ----  n-----si-f-------H      007.000.003  text/html   | - | aaaa-bbbb-cccc-dddd  | - | GET http://www.cnn.com/

In the above example, consecutive fields are delineated by alternating read and blue colors.

### Fields in the Access Log File

The fields comprising an access log entry are enumerated in the table below; the order in with the fields are listed in the table corresponds to the order as they appear in the access log entry.

| Field | Description |
| --- | --- |
| Time Stamp | Time and date the entry was recorded. This corresponds to the time when the HTTP request was received by the Web Proxy service from the client. |
| Process ID | The process ID of the web proxy child process that handled the request. This entry should include a ‘p’ followed by the process ID integer number with no white space between the ‘p’ and integer, as reflected in the example. |
| Request Size | The size, in bytes, of the HTTP request received from the client. |
| Connection ID | For EBO connections, this is the X-EBO-Req Key. For clientless connections, this is ‘0’. |
| Request Index | For EBO connections, this is ‘1’. |
| Client IP | Source/Client IP address. |
| Client Port | Source/Client port. |
| Server-Local Port | Local port used by the Web Proxy to connect to end-server. |
| Incoming Interface | The ID of the interface on which the HTTP request was received from the client. |
| Server IP | Destination/Server IP address. |
| Server Port | Destination/Server port. |
| Outgoing Interface | The ID of the interface on which the HTTP request from the Web Proxy was sent out. |
| Transaction Time | This is the total transaction time, in milliseconds, to service the request. If the content is retrieved from the cache, then transaction time corresponds to the time it takes to retrieve data from the cache. If the content is obtained directly from the content server, then the transaction time includes the time needed to get the data from end server and perform any applicable optimizations. |
| Compression Time | Time, in milliseconds, to compress the object. If the object is retrieved from the cache, then it is the time it originally took to compress the object. |
| Original Size | Size, in bytes, of the original object. This size includes the HTTP headers sent by the content server when the object is received by the Web Proxy. |
| Optimized Size | Size, in bytes, of the object and HTTP headers after optimization is performed by the Web Proxy. This is the number of bytes sent to the client from the Web Proxy. |
| Compression Percentage | Percentage reduction in size as a result of optimization. |
| Cache Status Code | Specifies the type of cache hit or miss (the Cache Codes list below enumerates the status codes, and explains the meaning of each code). |
| HTTP Status Code | Specifies the HTTP status code returned from the server (the HTTP Codes list below enumerates the status codes, and provides additional details). |
| Optimization Flag 1 | N/A |
| Optimization Flag 2 | N/A |
| Optimization Flag 3 | N/A |
| Optimization Flag 4 | N/A |
| Optimization Flag 5 | N/A |
| Optimization Flag 6 | N/A |
| Optimization Flag 7 | ‘E’ for EBO connections. ‘-‘ for non-EBO connections |
| Optimization Flag 8 | N/A |
| Optimization Flag 9 | ‘U’ if EBO Stream Compression was applied, ‘-‘ if EBO Stream Compression was not applied. |
| Optimization Flag 10 | N/A |
| Optimization Flag 11 | 'P' if the Web proxy included an X-EBO-Pred header in the response. 'p' if this objected included the X-EBO-Req Predicted Request flag. '-' No object prediction performed |
| Optimization Flag 12 | N/A |
| Optimization Flag 13 | N/A |
| Optimization Flag 14 | N/A |
| Optimization Flag 15 | N/A |
| Optimization Flag 16 | N/A |
| Selective Handling Flag 1 | N/A |
| Selective Handling Flag 2 | N/A |
| Selective Handling Flag 3 | N/A |
| Selective Handling Flag 4 | N/A |
| Selective Handling Flag 5 | N/A |
| Selective Handling Flag 6 | This flag indicates selective disabling of optimizations actions taken as a result of bmsdo configuration. Possible values for this flag and their meanings are shown below. ‘E’ no EBO ‘A’ no optimizations ‘-‘ no SDO performed |
| General Flag 1 | N/A |
| General Flag 2 | N/A |
| General Flag 3 | N/A |
| General Flag 4 | N/A |
| General Flag 5 | N/A |
| General Flag 6 | N/A |
| General Flag 7 | N/A |
| General Flag 8 | N/A |
| General Flag 9 | N/A |
| General Flag 10 | N/A |
| General Flag 11 | N/A |
| General Flag 12 | N/A |
| General Flag 13 | N/A |
| General Flag 14 | N/A |
| Generic Flag 15 | N/A |
| General Flag 16 | N/A |
| EBO License Info | This field specifies the EBO related information needed for UCR licensing. There are three fields separated by ‘.’ Delimiters. br_type an integer field (zero padded 3 character) indicating the browser vendor supporting EBO ebo_type an integer field (zero padded 3 character) indicating the type of EBO implemented by the browser for this transaction dev_class an integer field (zero padded 3 character) indicating the device class that the EBO browser is running on |
| Content Type | This field indicates the MIME type of content returned from the server, in format type/subtype. If the MIME type is unknown, then a ‘-‘ is used instead. |
| MSISDN | N/A |
| Session-ID | N/A |
| User Agent | This field displays user agent string if corresponding field is configured in webproxy.conf file. ‘string’ if log_user_agent_str is set to ON (the default) in webproxy.conf ‘-‘ if log_user_agent_str is set to OFF in webproxy.conf. This is the default setting. Note the user agent string will be printed up to a maximum of 100 characters in the web proxy access log. |
| HTTP Method | This field specifies the HTTP method used in the request coming from the client. |
| Request URL | The URL in the HTTP request received from the client. For pre-fetch requests generated by the Web Proxy in the context of MOP optimization, this is the pre-fetch request URL. The URL by default is logged up to 512 bytes in length. The query string (starting with delimited ‘?’) can also be left out if so configured. |

### Cache Codes

The following table enumerates the various Cache Codes used in the Access Log file.

| MEM_HIT | Cache HIT - object was found in the RAM memory cache |
| --- | --- |
| HIT | Cache HIT - the object requested was found in the disk cache |
| MISS | Cache MISS - the object requested was not found in the cache |
| REF_HIT | Refresh HIT - copy in cache was revalidated by the OSN because the aged based calculation indicated that the cache entry had expired. This occurs when a client requests an object that is in the cache, but the age of the cache entry is such that the OSN must ensure it is up-to-date. |
| REF_MISS | Refresh MISS - copy in cache was outdated, so a new copy was retrieved, and sent to the client. This covers both an expired cache entry and those that were updated as a result of an If Modified Since (IMS) HTTP request. |
| CLNT_REF_MISS | Client forces retrieval of a new copy of the object from web server. |
| IMS_HIT | “If Modified Since” HIT - client asked for an object checking if it had been modified after a certain time. The OSN verifies with the server that the object in cache had not been modified since the specified time. Therefore, the response was served from the cache to the client. |
| SWAP_MISS | Cache MISS because the cached object was swapped out to make room for other more recently requested objects. The Web Proxy removes objects from the cache based on an LRU (least recently used) policy. |
| NEG_HIT | Cache HIT for a URL that has given an error recently. The OSN servers the error for a configurable amount of time. |
| DENIED | Access to the requested URL has been denied by the OSN’s Web Proxy. |

### HTTP Status Codes

The HTTP status codes used in the Access Log file are, for the most part, same as the standard codes as defined in RFC 2616. The first digit of the code conveys the overall meaning, while the last two digits describe the condition in more detail. The following table enumerates the HTTP status codes used in the Access Log file.

| 000 | This is a Web Proxy specific code and means that the transaction was incomplete. For example, this code is used when the user cancels aborts the request prematurely. Another use of this code is when the server terminates the connection unexpectedly. |
| --- | --- |
| 1xx | Informational - the final outcome of request is not known yet, however it has not been refused either. The actual status code will be sent once the processing of the command progresses further. |
| 2xx | Successful - the request was received, and processed successfully. |
| 3xx | Redirection - this type of status code is used to redirect a client to another URL, and to notify if a cached copy is still up-to-date. |
| 4xx | Client Error – this type of status code is used when the request could not be serviced due to a client error. |
| 5xx | Server Error – this type of status code is used when the request could not be serviced due to a server error. |
| 6xx | Bad Headers – this indicates that the OSN observed malformed HTTP headers |

## Information Log File

### Fields in the Information Log File

The Information Log file records informational, warning, and error messages generated by the Web Proxy service. This file is often referred to as the Info Log file. Like the Access Log file, there is a single Info Log file irrespective of the number of Web Proxy processes. Since there can be more than one Web Proxy processes running on an OSN, the process ID is included in the Info Log file entries.

A sample entry in the Info Log file is shown below.

2003/05/27 00:00:19| 6690:1 | commSetSelect: FD 11 type 1

As shown above, each entry has three fields; fields are delineated using the ‘|’ character.

The first field is the time when then this entry was made.

The second field comprises of the process ID and process number, in the format <process ID>:<process number>. The process ID is simply the Operating System assigned process ID for each of the Web Proxy processes. The process number corresponds to the child index of each of the Web Proxy processes. Note that when the Web Proxy service is started, a “parent” process is first stared that in turn spawns one or more child processes. The process number of the parent process is set to zero. The first child process is assigned a process number of one; the second child process is assigned a process number of two, and so on.

The third field in the Info Log file is a free form message – there is no specific format for this field.

### Controlling Log Messages

The Web Proxy provides a flexible mechanism to control which messages are written to the Info Log file. There are two dimensions to this control mechanism.

- The modules (or sections) that will log messages to the Info Log file can be selected via a configuration file entry.

- For each module, the verbosity level (i.e. log level) can be set anywhere in the range zero to ten, where zero means least verbose and ten means most verbose.

The modules and log level can be set via the configuration option “debug_options” in the Web Proxy’s configuration file; the default location for the configuration file is /opt/bmi/etc/webgrp/webproxy.conf. The syntax for this configuration option is shown below.

debug_options x1,y1 x2,y2 … xn,yn

As shown above, a pair of integer values is used for each module with a space character delineating the individual modules. The first value in each pair corresponds to the module number – there are 100 unique module numbers (0-99), though not all are currently used. The special keyword “ALL”, when used in place of a module number, means that all modules are to be selected for logging. The second value in the pair corresponds to the log level. The allowed range for the verbosity level is from 0 to 10. A log level of less than 0 is equivalent to 0, and a log level of greater than 10 is equivalent to a log level of 10.

The EBO info log section is 60.

## Fatal Log File

The Fatal Log is useful in diagnosing service affecting conditions in the Web Proxy. To this end, the Web Proxy maintains a circular queue of debug log messages. When the Web Proxy first encounters an unexpected condition that is deemed to be of a fatal nature, the Fatal Log file is created and the contents of the internal queue of debug log messages are printed to the Fatal Log file. If more fatal conditions are encountered, then additional log messages from the queue are appended to the Fatal Log file. The format of entries in the Fatal Log file is similar to that of entries in the Info Log file.

## Log File Rotation

The Access and Info log files are rotated periodically; in the default installation of the OSN, log files are rotated once every day.  After the first rotation, the web_info.log is moved to web_info.log.0 and new log messages are again written to web_info.log.  After the second rotation, web_info.log.0 is first moved to web_info.log.1, and then web_info.log is moved to web_info.log.0, and the new log messages are once again written to web_info.log. Each time log files are rotated, the last number on each file is increased by one and again the new content is written to web_info.log.  This will continue up until the maximum number of log files is reached (as specified in the web proxy configuration file, webproxy.conf; the default value is 10), after which the highest number will be overwritten.  The same log rotation method is implemented for the Access and Info Log files.

# Web Proxy Configuration Options

Use the following commands to turn EBO on and off:

sudo /opt/bmi/bin/bmconfig –o ebo,ebo_address=1.2.3.4,ebo_port=80

sudo /opt/bmi/bin/bmconfig –o noebo

Use this command to check the current EBO configuration:

sudo /opt/bmi/bin/bmconfig –l | grep ebo

You can also directly change the webproxy’s configuration settings by editing the file /opt/bmi/etc/webgrp/webproxy.conf and then restarting the webproxy using this command:

sudo /opt/bmi/bin/bmproc --restart web

The most interesting configuration settings for EBO debugging are:

- enable_ebo on

- ebo_address 1.2.3.4

- ebo_port 80

- enable_ebo_compression on

- ebo_conn_idle_timeout 30 seconds

- cache on

- debug ALL,1 60,5

- enable_explicit_mode on

- http_listen_info explicit 3129

sudo access is also provided for the snoop command.