- EBO Generation 3 Specification - Working Implementation v1 Updated July 30, 2026 Prepared by: Fred Koopmans Bytemobile, Inc. 2025 Stierlin Court Mountain View, CA 94043, USA Tel: 1-650-641-7700 Fax: 1-650-641-7701

# Revision Control

| Version # | Date | Author | Changes |
| --- | --- | --- | --- |
| DRAFT | 07/28/2006 | Fred Koopmans | - First external draft |
| DRAFT | 08/25/2006 | Fred Koopmans | - Added EBO Authentication |
| DRAFT | 08/31/2006 | Michael Youssefmir | - Reordered and Modified to create normative specification |
| DRAFT | 09/06/2006 | Fred Koopmans | - Editing |
| DRAFT | 01/15/2007 | Fred Koopmans | - Added image modification flags |
| WI v1 | 01/22/2007 | Fred Koopmans | - Changed DRAFT to Working Implementation v1 |

Revision Control	2

1	Preliminaries	5

1.1	Terminology	5

1.2	Introduction	5

1.3	EBO-3 Overview	5

1.4	Supported Architectures	6

2	Required EBO-3 Features	6

2.1	EBO-3 Detection and Configuration Exchange	6

2.1.1	Optimizer Detection	6

2.1.2	Browser Detection and Configuration Exchange	8

2.2	EBO-3 Dynamic Interleaving	9

2.2.1	Out-of-Order Responses	9

2.2.2	Message Types	10

2.2.3	Connection Handling	12

2.3	EBO-3 Stream Compression	12

2.3.1	Stream Compression Enabled	12

2.3.2	Stream Compression Disabled	13

2.3.3	Memory Requirements	13

2.4	EBO-3 Prediction	13

2.4.1	EBO-3 Prediction Requirements	14

2.5	EBO-3 Selective Disablement of Optimization	15

2.5.1	EBO-3 Selective Disablement of Optimization Requirements	15

2.6	EBO-3 Proxy Support	15

2.7	EBO-3 Authentication	15

2.7.1	Authentication Process	16

2.7.2	Authentication Requirements	17

3	Optional EBO-3 Features	18

3.1	Optimization Personalization	18

3.2	Selective Image Retrieval	19

3.3	Image Resizing	19

4	EBO-3 Header Definitions	19

4.1	The X-EBO-UA header	20

4.1.1	X-EBO-UA Requirements	20

4.1.2	X-EBO-UA Parameters	20

4.2	The X-EBO-Opt header	20

4.2.1	X-EBO-Opt Requirements	21

4.2.2	X-EBO-Opt Parameters	21

4.3	The X-EBO-Info header	22

4.3.1	X-EBO-Info Requirements	22

4.3.2	X-EBO-Info Parameters	22

4.4	The X-EBO-Req header	24

4.4.1	X-EBO-Req Requirements	24

4.4.2	X-EBO-Req Parameters	25

4.5	The X-EBO-Resp header	25

4.5.1	X-EBO-Resp Requirements	25

4.5.2	X-EBO-Resp Parameters	26

4.6	The X-EBO-Pred header	26

4.6.1	X-EBO-Pred Requirements	26

5	License Enforcement Parameters	27

Appendix A: EBO Authentication Scenarios	27

# Preliminaries

## Terminology

The word shall is used to indicate mandatory requirements strictly to be followed in order to conform to the EBO-3 Specification and from which no deviation is permitted (shall equals is required to).

The word should is used to indicate that among several possibilities one is recommended as particularly suitable, without mentioning or excluding others; or that a certain course of action is preferred but not necessarily required; or that (in the negative form) a certain course of action is deprecated but not prohibited (should equals is recommended that).

The word may is used to indicate a course of action permissible within the limits of the EBO-3 Specification (may equals is permitted to).

The word can is used for statements of possibility and capability, whether material, physical, or causal (can equals is able to).

## Introduction

This document defines Bytemobile’s Embedded Browser Optimization Generation 3 (EBO-3) including protocol, required features, and optional features.  The primary components of EBO-3 are as follows:

- Automatic discovery and configuration exchange between EBO enabled browsers and EBO enabled optimizers.

- Advanced Bytemobile client/server HTTP optimizations, including Macara Dynamic Interleaving, Universal Application Compression, and Client Object Prediction.

- Licensing, Two-way authentication, and Selective Disablement of Optimization

With this feature set, EBO-3 improves the end-user mobile web browsing experience and offers the mobile operator a rich set of configuration options.

## EBO-3 Overview

The basic idea of EBO is for the browser to open a normal, unoptimized connection to issue its first request of a given browsing session.  If an EBO optimizer is present in the network, it includes a header in the response that indicates its existence and its optimization capabilities.  The browser detects this header, closes the unoptimized connection, and opens an optimized connection to finish the download.  On the optimized connection, or “EBO connection”, the browser and optimizer each perform various optimizations that are agreed upon during the initial discovery process.  An “EBO connection” is defined as any HTTP connection in which the browser knows that it is connected to an EBO optimizer, and the optimizer knows that it is connected to an EBO browser.

Optimization is achieved while maintaining the following operational goals:

- EBO is transparent to the end user and the content servers.

- EBO is self-configuring.  There are no manual configuration or installation steps required on either the handset or the optimizer.

- An EBO browser behaves normally when an EBO optimizer is not present, with no impact to end-user experience.

- An EBO optimizer behaves normally and provides standard non-EBO optimization for non-EBO browsers, with no impact to end-user experience.

- EBO is either transparent to any network elements positioned between the browser and the optimizer or provides a mechanism to bypass certain content that needs to be inspected without optimization.  For example, some mobile operator networks include a transparent billing node between the browser and optimizer that only supports standard HTTP 1.1 or WAP2 formats.

- EBO does not require the optimizer to store any persistent state between TCP connections.

Note: Operation of EBO requires that no network elements (such as caches) sit between the browser and the optimizer modifying the browser’s HTTP data in any way.

## Supported Architectures

EBO supports the following three mobile network configurations:

a.

b.

c.

Browser

Optimizer

Internet

Browser

Optimizer

Internet

WAP2 Gateway

Browser

Optimizer /

WAP2 Gateway

Internet

Figure 1: Supported Architectures.

# Required EBO-3 Features

This section defines the EBO-3 required features that shall be implemented by all EBO-3 compliant browsers and optimizers.

## EBO-3 Detection and Configuration Exchange

### Optimizer Detection

An EBO-3 browser shall automatically detect the presence of an EBO optimizer as follows:

- At the start of discovery, the browser shall begin by issuing the first request normally.  That is, the browser performs a DNS lookup for the desired URL, opens a TCP connection to the corresponding origin server’s IP address, and issues the first request.  Or, if the browser is using a proxy, it instead opens a TCP connection to the proxy’s IP address and issues the first request.  In either case, this first connection of the browsing session is an unoptimized connection.  Additionally, the browser shall include an “X-EBO-UA” HTTP header in the request to advertise that it supports EBO.

- If this TCP connection is intercepted by an optimizer, the optimizer first satisfies the request by retrieving the requested object either from its cache or from the origin server.  Next, if the optimizer detects the X-EBO-UA header in the request, it shall include an “X-EBO-Opt” HTTP header in the response that identifies the presence of the optimizer.  This header also specifies exactly which optimizations are supported by the optimizer.

- When the browser receives the first response on an unoptimized connection, it shall check the response headers to determine if the X-EBO-Opt header is present.

  - If X-EBO-Opt is not present, the browser shall continue loading the page normally over unoptimized connections.  In this case, the browser shall repeat the discovery process intermittently in the future.

  - If X-EBO-Opt is detected, the browser has successfully confirmed that an optimizer is present in the network.  The browser shall then close the unoptimized connection and open an EBO connection to finish loading the page and any subsequent pages for the rest of the browsing session.

- Once an optimizer has been discovered, the browser shall reconfirm the optimizer’s presence and supported optimizations on each new EBO connection.  This is necessary to detect any changes to the optimizer.  If the configurations set forth in the X-EBO-Opt header in the first response on each EBO connection changes in any way from previously received values, the browser shall close the EBO connection and return to step 1 of the discovery process.

This discovery/confirmation procedure is illustrated in Figure 2.  The browser initially opens an unoptimized connection (TCP Conn 1) and sends the request for url1 with the X-EBO-UA header.  The optimizer intercepts this request, detects the X-EBO-UA header, and includes the X-EBO-Opt header in the response.  The browser detects the X-EBO-Opt header, opens an EBO connection (TCP Conn 2), and sends the requests for url2 and url3.  Finally, the browser verifies that the EBO connection was intercepted by the optimizer by confirming that the X-EBO-Opt header was included in the response for url2.

GET url1 HTTP 1.1

X-EBO-UA:

…

HTTP 1.1 200 OK

X-EBO-Opt:

…

GET url2 HTTP 1.1

X-EBO-Info:

…

GET url3 HTTP 1.1

…

HTTP 1.1 200 OK

X-EBO-Opt:

…

HTTP 1.1 200 OK

…

Browser

Optimizer

Origin Server

Phase 1:

EBO Discovery

(No Optimization)

Phase 2:

EBO Optimization:

TCP Conn 1

…

TCP Conn 2

…

Figure 2: EBO Optimizer Detection

Note: The discovery process (i.e., phase 1) occurs only once at the beginning of each browsing session.  However, the confirmation process (i.e., phase 2) occurs on every new EBO connection.

Note: The optimizer includes the X-EBO-Opt header in the first response on each connection.  Thus, the browser only needs to inspect the first response for this header.

### Browser Detection and Configuration Exchange

An EBO-3 browser shall identify EBO connections and specify which optimizations are to be performed as follows:

- Each time the browser opens a new EBO connection, it shall include an “X-EBO-Info” HTTP header in the first request on that connection.  This informs the optimizer that this is an “EBO connection”, as opposed to an unoptimized connection.

- The browser shall also indicate the requested optimizations in the X-EBO-Info header by selecting from those optimizations advertised in the X-EBO-Opt header.

- If the browser does not support a particular optimization advertised by the optimizer, it shall not include this optimization parameter in X-EBO-Info header.  This informs the optimizer that it shall not perform this optimization.

- Conversely, if one of the browser’s supported optimizations is not advertised in the X-EBO-Opt header, the browser shall not include this optimization parameter in the X-EBO-Info header.  Furthermore, the browser shall not perform this optimization.

This negotiation procedure is illustrated in Figure 3.  In this example, the browser supports optimizations O1 and O2 and the optimizer supports optimizations O1 and O3.  As part of the discovery procedure, the browser inspects the X-EBO-Opt header and learns that the optimizer supports O1 and O3.  Next, the browser determines that only O1 is also supported by the optimizer and therefore the browser only includes O1 in the X-EBO-Info header.  Thus, for this particular EBO connection (TCP Conn 2), both the browser and the optimizer know that only optimization O1 is supported.

GET url1 HTTP 1.1

X-EBO-UA:

…

HTTP 1.1 200 OK

X-EBO-Opt: O1, O3

…

GET url2 HTTP 1.1

X-EBO-Info: O1

…

GET url3 HTTP 1.1

…

HTTP 1.1 200 OK

X-EBO-Opt: O1, O3

…

HTTP 1.1 200 OK

…

Browser:

Supports O1, O2

Optimizer:

Supports O1, O3

Origin Server

Phase 1:

EBO Discovery

(No Optimization)

Phase 2:

EBO confirmation

TCP Conn 1

…

TCP Conn 2

…

Figure 3: EBO Browser Detection and Configuration Exchange

If the browser detects any kind of error on an EBO connection, it shall close the EBO connection, reset any saved EBO state, and go back to step 1 of the EBO discovery process.  Possible EBO connections errors include:

- Not receiving the X-EBO-Opt header in the first response on an EBO connection.

- Receiving a different X-EBO-Opt header than was originally received during the discovery process.

- Receiving a premature TCP reset in the middle of an HTTP response on an EBO connection.

- Failure to open a TCP connection to the optimizer.

Closing of an idle EBO connection by the optimizer shall not constitute an error.  In such a case, the browser may open a new EBO connection to the optimizer per the procedure above.

## EBO-3 Dynamic Interleaving

An EBO-3 browser shall implement Dynamic Interleaving.

EBO-3 requires a request/response procedure known as Dynamic Interleaving between the browser and the optimizer.  The primary benefit of Dynamic Interleaving over HTTP 1.1 is that it allows the optimizer to send the HTTP responses out of order, instead of requiring the optimizer to match the order in which the browser generated the requests.  The Dynamic Interleaving protocol also defines some control messages that are needed to handle a few error conditions and interoperability requirements.

### Out-of-Order Responses

An EBO-3 browser shall support out-of-order responses as follows:

- The browser shall send all requests to the optimizer as fast as it can on a single EBO connection.

- The browser shall assign a unique request key to each HTTP request and include this in a special HTTP request header.  The browser shall also record this mapping of request keys to URLs for later use.

- The optimizer shall receive the EBO requests, download all responses in parallel from the multitude of origin servers, and then determine an (optimal) response order based on content type, size, content server speed, etc.  The optimizer shall then send the responses one at a time over the EBO connection in the selected order.

- The optimizer shall include the browser-assigned request key in each response.  The browser shall accept responses in arbitrary order and shall associate the response content with the requested URL.

- Upon selecting a particular response to send, the optimizer shall send the entirety of this response before starting the next response.  In other words, Dynamic Interleaving in EBO-3 does not support response interruption.

This behavior is depicted in Figure 4 below.  In this example, the browser issues four requests on the EBO connection in the order r1, r2, r3, r4.  The optimizer forwards each of these requests to the corresponding origin servers and downloads the responses in parallel.  The optimizer then selects the best response ordering, in this case R3, R2, R4, R1, and sends the responses to the browser.

r4  r3  r2  r1

R3  R2  R4  R1

Browser

Optimizer

Server 1

Server 2

Server 3

Server 4

r1  R1

r2  R2

r3  R3

r4 R4

EBO Connection

Unoptimized,

HTTP 1.1 Connections

Figure 4: Dynamic Interleaving Example.

### Message Types

An EBO-3 browser shall support the following four distinct Dynamic Interleaving message types:

- request

  - Other than the addition of the EBO headers, a Dynamic Interleaving request shall be no different than any other standard HTTP request.

  - Request messages shall always be HTTP 1.1 and shall include a GET, HEAD, POST, or PUT method.  Other request methods (e.g., CONNECT) shall not be sent on EBO connections.

  - If the browser is using a proxy, request messages shall include a fully qualified URL (e.g., “http://hostname/url” instead of “/url”).  The optimizer shall forward these requests as they are to the proxy address provided in the X-EBO-Info header.

  - If the browser is not using a proxy, request messages shall include a Host header and shall not include a fully qualified URL (e.g., “/url” instead of “http://hostname/url”).  The optimizer shall use the value of the Host header to forward the request to the correct content server.

- response

  - Other than the addition of the EBO headers, a Dynamic Interleaving response shall be no different than any other HTTP response.

  - Response messages shall always be HTTP 1.1 and may be chunked transfer encoded by the optimizer.

  - The browser shall be capable of accepting responses in a different order than that in which the corresponding requests were sent.

  - The browser shall accept EBO responses that may include up to 8192 bytes of response headers.

  - The browser shall accept EBO responses that may include a single HTTP header of up to 2096 bytes.

    - Note: These last two requirements are necessary to support EBO-3 Prediction.

- abort

  - The browser may send an abort message to cancel a previously requested object without closing the EBO connection.  For example, if a JavaScript dynamically rewrites a portion of the page such that one of the previously requested images is no longer needed, the browser may send an abort message to cancel this transaction.  However, if the browser needs to cancel many requests at once (e.g., if the user presses stop or clicks a link while a page is still loading), the browser shall instead close the current EBO connection and open a new EBO connection instead of aborting each of the objects individually.

  - When the optimizer receives an abort message, it shall cancel the corresponding response (as identified by the X-EBO-Req Key parameter) if it has not already started sending the response data.  If the optimizer has already sent or has already started sending the response data, it shall ignore the abort message.

  - If the browser starts to receive a response that it has previously aborted, it shall take the following action:

    - If the response includes a Content-Length that is smaller than 20 KB, the browser shall allow the download to complete and silently discard the data.

    - If the response does not include a Content-Length, or the Content-Length is greater than 20 KB, the browser shall close the current EBO connection, open a new EBO connection, and reissue any requests that are still needed.

  - An abort message shall be a valid HTTP request message, but should contain as few bytes as possible.  For example,

GET / HTTP 1.1

X-EBO-Req: Key=7, Flags=1

- bypass

  - The optimizer shall send a bypass message to the browser if it is unable to service a particular request on an EBO connection.  For example, the optimizer may send a bypass message to the browser if the response content is large enough that it may block other objects on the EBO connection (e.g., 200 KB).

  - A bypass message shall be a valid HTTP response message, but should contain as few bytes as possible.  For example,

HTTP 1.1 400 Bad Request

X-EBO-Resp: Key=7, Flags=1

Content-Length: 0

  - When the browser receives a bypass message, it shall open an unoptimized connection directly to the origin server and reissue the request.

  - The browser shall not include any EBO headers (e.g., X-EBO-UA) on bypassed connections.

  - The optimizer may also include a bypass rule in a bypass message that specifies a bypassed domain, hostname, or partial URL.

  - When the browser receives a bypass rule, it shall save the rule in memory (i.e., not in persistent storage).  Additionally, before issuing each subsequent request, the browser shall check the desired URL against the saved bypass rules from that browsing session.

    - If the desired URL does not match any of the saved bypass rules, then the browser shall issue the request normally on the EBO connection.

    - If the desired URL matches one of the saved bypass rules, the browser shall bypass the request.  That is, the browser shall perform a DNS lookup, open a TCP connection directly to the origin server, and issue the request.

  - A bypass message shall be a valid HTTP response message, but should contain as few bytes as possible.  An example bypass message with a hostname bypass rule could be,

HTTP 1.1 400 Bad Request

X-EBO-Resp: Key=7, Flags=1, Bypass=abc.com

Content-Length: 0

  - An example bypass message with a URL bypass rule could be,

HTTP 1.1 400 Bad Request

X-EBO-Resp: Key=7, Flags=1, Bypass=abc.com/foo/bar

Content-Length: 0

### Connection Handling

An EBO-3 browser shall support the following EBO connection edge cases:

- If the browser closes an EBO connection, the optimizer shall discard any remaining requests and responses associated with that connection.

- If the optimizer closes an interleaved connection with requests outstanding, the browser shall reissue all outstanding requests on bypassed unoptimized connections.  This is necessary to prevent an infinite loop if the optimizer encounters a deterministic problem while servicing one of the requests.

- If the browser stops receiving data from the optimizer while requests are outstanding, the browser shall wait up to 60 seconds before closing the EBO connection and reissuing all outstanding requests on unoptimized connections.

- If the browser receives a large EBO response (e.g., greater than 200 KB) with no Content-Length header and has other outstanding requests that are being blocked by the large response, the browser shall open a new EBO connection and reissue the blocked requests.  The browser shall then close the original EBO connection as soon as the large response is completed without downloading any subsequent responses.  This behavior prevents a single large object (e.g., a video or file download) from blocking the rest of the page from loading.

- The optimizer may close idle EBO connections after an appropriate timeout, often selected by the mobile operator.  The browser should not initiate an idle EBO connection close except when necessary (e.g., when exiting).

## EBO-3 Stream Compression

An EBO-3 browser shall support EBO Stream Compression by performing ZLIB inflate and deflate on EBO connections.  The browser shall also allow the optimizer to either enable or disable this feature.

### Stream Compression Enabled

When stream compression is enabled, the following requirements apply:

- The optimizer shall advertise that Stream Compression is enabled in the X-EBO-Opt header.

- The browser shall deflate all request data before writing to the TCP layer, including any content data provided in POST or PUT requests.

- The browser shall attempt to inflate all response data after reading from the TCP layer.

  - If the inflate succeeds, the browser shall process the EBO responses normally.

  - If the inflate fails, the browser shall close the EBO connection and go back to step 1 of the EBO discovery process.

- If the browser receives a 400 Bad Request response with an X-EBO-Opt header indicating that Stream Compression is now disabled, the browser shall either reissue the request on a new EBO connection with Stream Compression disabled or go back to step 1 of the EBO discovery process.

- The optimizer shall attempt to inflate request data received on connections destined to the EBO address.

  - If the inflate succeeds, the optimizer shall process the EBO requests normally and also deflate the response data before writing to the TCP layer.  Additionally, the optimizer should not send any content-encoded EBO responses (e.g., no gzip or deflate encoding).

  - If the inflate fails, the optimizer shall reject the browser’s connection by sending an uncompressed 400 Bad Request response and closing the connection.

### Stream Compression Disabled

When stream compression is not enabled, the following requirements apply:

- The optimizer shall advertise that Stream Compression is not enabled in the X-EBO-Opt header.

- The browser shall not deflate the request data.

- The browser shall not attempt to inflate the response data.

- If the browser receives a 400 Bad Request response with an X-EBO-Opt header indicating that Stream Compression is now enabled, the browser shall either reissue the request on a new EBO connection with Stream Compression enabled or go back to step 1 of the EBO discovery process.

- The optimizer shall attempt to inflate request data received on connections destined to the EBO address.

  - If the inflate succeeds, the optimizer shall reject the connection by sending a deflated 400 Bad Request response and closing the connection.

  - If the inflate fails, the optimizer shall process the EBO requests normally and shall not deflate the response data.  Additionally, the optimizer may send content-encoded EBO responses if permitted by the browser.

### Memory Requirements

In order to minimize the amount of additional memory and CPU cycles required to perform stream compression, an EBO-3 browser should adjust the ZLIB deflate configuration parameters as follows:

- Widow Bits: Default Value = 15, Recommended Value = 12

- Memory: Default Value = 8, Recommended Value = 4

- Optimization level: Default Value = 6, Recommended Value = 1

These settings reduce the amount of memory required to perform ZLIB deflate from 256 KB to 24 KB.

For response data, the optimizer shall deflate the data stream using a Window Bits setting less than or equal to 15.  For the browser to properly inflate this data stream, the browser shall use a Windows Bits setting equal to 15 when inflating, which requires a one time allocation of approximately 44 KB of memory.

Note: More information on the memory requirements of ZLIB inflate and deflate is available at http://www.zlib.net/zlib_tech.html.

## EBO-3 Prediction

This EBO optimization allows the optimizer to include a list of URLs in a response object that the browser can start requesting immediately, before parsing and executing the page’s HTML, JS, and CSS content.  The browser proactively requests these objects on the assumption that it will later need the objects, but before it knows for certain that it needs all of the objects.  The vast majority of the time, this optimization yields a substantial performance improvement by parallelizing the download of the page with the execution of the page (i.e., parsing HTML, executing JavaScript, applying CSS, etc.) without requiring any extra downloads.  However, like all prediction based algorithms, there are a few edge cases in which the browser may need to re-request an object that was not properly requested the first time.

### EBO-3 Prediction Requirements

An EBO-3 browser shall support EBO-3 Prediction.

- The browser shall advertise its support for Prediction in the X-EBO-Info header to inform the optimizer that it is okay to include predicted URLs.

- When the optimizer intercepts HTML or CSS content, it shall scan through the content to extract the URLs the browser it likely to request.  The optimizer shall then include these URLs in a special HTTP header in the response.

  - This header is named “X-EBO-Pred” and shall include a list of fully qualified URLs separated by spaces. For example,

X-EBO-Pred: http://hostname1/url1 http://hostname1/url2

  - The optimizer may include more than one X-EBO-Pred header, all of which shall be processed by the browser in the order in which they are received.  For example,

X-EBO-Pred: http://hostname1/url1 http://hostname1/url2

X-EBO-Pred: http://hostname2/url3 http://hostname2/url4

  - Note: The browser shall be prepared to receive a single X-EBO-Pred header up to 2048 bytes long.

- When the browser receives a response, it shall first process all Set-Cookie headers in that response and then check for one or more X-EBO-Pred headers.

- When the browser receives an X-EBO-Pred header, it shall generate a new HTTP request for each URL in the order in which the URLs are received.  Each predicted request shall also set the “Predicted Request” flag as described in section 4.4.2 to allow the optimizer to distinguish between predicted requests and non-predicted requests.

- The browser shall send predicted requests through the same HTTP control flow path as non-predicted requests.  This shall include the following:

  - If the requested object already exists in the browser’s cache and is fresh, the predicted request shall be cancelled.

  - If the requested object already exists in the browser’s cache and is stale, the browser shall include an If-Modified-Since header in the predicted request.

  - If the browser already has an outstanding request for the same object (predicted or non-predicted), the predicted request shall be cancelled.

  - If the requested object matches any of the browser’s existing cookies, the browser shall include these cookies in the predicted request.

- When the optimizer receives a predicted request, it shall process the request the same as a non-predicted request.

- When the optimizer receives a predicted response, it shall cache the content while waiting for the object to be needed.  Predicted response content shall be cached in memory to optimize retrieval.

- When the browser needs to issue a non-predicted request (e.g., it parsed a URL from an HTML or CSS object), it shall first check to see if a predicted request has already been issued before sending this request.  If so, the browser shall wait for the predicted response to arrive instead of issuing a duplicate request.

- When the browser is ready to use a predicted response, it shall perform a final check before using the content:

  - If the predicted response is cacheable (either publicly or privately) and has not yet expired, the content may be used.

  - If the predicted response is not cacheable or has already expired, the browser shall check if the cookies used in the predicted request are the same as the cookies that would be used if the request were reissued.  This is necessary to determine if some JavaScript code has created or modified one of the cookies associated with this URL.  If it has, and the object is not cacheable, then the browser shall discard the predicted response and reissue the request.

    - Note: This scenario is not common, but it is nonetheless necessary to handle this edge-case to ensure that prediction does not break any websites.

- If the browser does not find a need for a predicted response by the time the webpage finishes downloading, the browser may discard the predicted response.

- As a final safeguard, if a single predicted response ever exceeds 200 KB before the browser has found the need for this object, the browser shall abort the predicted request by closing the EBO connection and reissuing all remaining requests on a new EBO connection.

## EBO-3 Selective Disablement of Optimization

This EBO feature allows the operator to disable EBO optimization for specific web content.  The primary purpose of this feature is to allow a network element that resides between the browser and the optimizer to inspect the unmodified HTTP traffic.  For example, an operator may want to disable EBO optimization for its portal content such that it’s billing node can charge appropriately for access to this content.

The EBO-3 implementation of Selective Disablement of Optimization (called SDO for short) allows the operator to create a list of domains and URLs that shall not receive EBO optimization.  An example domain-type SDO rule is “abc.com”.  An example URL-type SDO rule is “abc.com/foo/bar”.

### EBO-3 Selective Disablement of Optimization Requirements

If the browser sends an EBO request for a URL that matches one of the optimizer’s SDO rules, the optimizer shall respond by sending an EBO bypass response message including an appropriate bypass rule.  The browser shall process this bypass message as described in section 2.2.2.

## EBO-3 Proxy Support

If the user or operator has configured the browser to use a proxy server, the browser shall perform all EBO optimizations exactly as if it were not using a proxy and also communicate the proxy address to the optimizer in the X-EBO-Info header as described in section 4.3.2.

## EBO-3 Authentication

In order to protect all parties involved (i.e., Bytemobile, the browser company, the mobile operator, and the user), the EBO-3 protocol also includes two-way authentication.  This allows the browser to validate that the optimizer is really a Bytemobile optimizer before connecting to it.  This protects the browser from connecting to a malicious proxy or content server pretending to be an EBO optimizer.  This also protects the browser from unknowingly connecting to an EBO optimizer from another optimization company with which it does not have a business agreement.  Conversely, this allows the optimizer to validate that the browser is really an EBO browser for which the mobile operator has purchased EBO optimization.  This protects the mobile operator from being over-charged for unauthorized EBO browsers or a malicious application pretending to be an EBO browser.

The process of EBO Authentication is similar to that of the CHAP standard described in RFC 1994.  In short, the authenticator presents a challenge request to the peer.  The peer uses a shared secret to manipulate the challenge request and sends the resulting challenge response back to the authenticator.  If the peer’s response matches the authenticator’s expected response, then the authenticator has successfully authenticated the peer’s identity.

### Authentication Process

On non-EBO connections,

- The browser shall authenticate the optimizer’s identity before opening an EBO connection.  This browser shall do this by including an authentication challenge in the X-EBO-UA header.  This is the “Browser Challenge Request”, or “BCReq”.

- If an EBO optimizer is present in the network, it shall include its authentication response in the X-EBO-Opt header.  This is the “Browser Challenge Response, or “BCResp”.

- If the optimizer provides an incorrect challenge response, the browser shall ignore the X-EBO-Opt header and continuing loading the page without EBO optimization.

This process is illustrated in Phase 1 of Figure 5.

On EBO connections,

- The browser shall re-authenticate the optimizer’s identity to ensure that it has connected to a valid optimizer.  The browser shall do this by including its authentication challenge request in the X-EBO-Info header and then inspecting the optimizer’s challenge response in the X-EBO-Opt header.

- If the optimizer provides an incorrect challenge response, the browser shall disconnect from the optimizer and go back to step 1 of the EBO discovery process.

- Additionally, the optimizer shall include its own authentication challenge request in the X-EBO-Opt header.  This is the “Optimizer Challenge Request”, or “OCReq”.

- The browser shall compute its authentication response to the optimizer’s challenge and include it in a second X-EBO-Info header in its next request.  This is the “Optimizer Challenge Response”, or “OCResp”.

- If the optimizer does not receive a challenge response in a timely fashion (as described in section 2.7.2.4) or it receives an incorrect challenge response, the optimizer shall send a 401 Unauthorized response and terminate the connection.  In this case, the optimizer shall not record a license count against this failed connection.

This process is illustrated in Phase 2 of Figure 5.

GET url1 HTTP 1.1

X-EBO-UA: Challenge1

…

HTTP 1.1 200 OK

X-EBO-Opt: Response1

…

GET url2 HTTP 1.1

X-EBO-Info: Challenge2

…

HTTP 1.1 200 OK

X-EBO-Opt: Resonse2, Challenge3

…

Browser

Optimizer

Origin Server

Phase 1:

EBO Discovery with Authentication

(No Optimization)

Phase 2:

EBO Optimization with Authentication

TCP Conn 1

…

TCP Conn 2

…

GET url3 HTTP 1.1

X-EBO-Info: Response3

…

HTTP 1.1 200 OK

…

Figure 5: EBO Authentication

### Authentication Requirements

#### Shared Secrets

Each EBO-3 browser type shall have an associated Browser ID (BID – see section 5).  Bytemobile shall define a unique 128-bit shared secret for each BID value.  Thus, there shall be a distinct shared secret for each EBO browser company, generation, and handset class.

The browser shall not store the shared secret in unencrypted form, either in a configuration file or in the browser’s source code.  Rather, the shared secret shall only be stored in an encrypted manner and then decrypted at runtime when needed.  The exact encryption algorithm is not specified in this specification.

#### Challenge Requests

In order to protect EBO Authentication from browser-side and optimizer-side replay attacks, each challenge request (i.e., BCReq and OCReq) shall include some form of randomness.  To satisfy these requirements, the challenge request shall be a randomly generated 128-bit integer.  Additionally, the browser shall ensure that the challenge request sequence is different for each browsing session, for example, by seeding the random number generator with the time of day.

#### Challenge Responses

In order to minimize the additional ROM, RAM, and CPU overhead required to perform EBO Authentication, the challenge responses shall be computed using the standard MD5 hash algorithm.

On non-EBO connections, the browser’s challenge response shall be computed as follows:

BCResp = MD5 (UpperCase (Shared SecretHigh16 “+” BID “+” BCReq “+” Shared SecretLow16))

where “Shared SecretHigh16” is the upper 16 bytes of the shared secret, “Shared SecretLow16” is the lower 16 bytes of the shared secret, and “BID” and “BCReq” are the values of the corresponding parameters in the X-EBO-UA header with any leading or trailing whitespace characters removed.  Additionally, the hash input shall be converted to upper case before computing the MD5 hash value.

As an example, suppose the browser sends the following header on a non-EBO connection:

X-EBO-UA: BID=1.3.0, BCReq=0123456789abcdef0123456789ABCDEF

Further suppose that the shared secret for BID 1.3.0 is

00000000000000001111111111111111

In this case, both the browser and the optimizer construct the challenge response by computing the MD5 hash of the string

0000000000000000+1.3.0+0123456789ABCDEF0123456789ABCDEF+1111111111111111

On EBO connections, the browser’s challenge response shall be computed as follows:

BCResp = MD5 (UpperCase (Shared SecretHigh16 “+” BID “+” UID “+” BCReq “+” Shared SecretLow16))

where “BID”, “UID”, and “BCReq” are the values of the corresponding parameters in the X-EBO-Info header with any leading or trailing whitespace characters removed.

On EBO connections, the optimizer’s challenge response shall be computed as follows:

OCResp = MD5 (UpperCase (Shared SecretHigh16 “+” Address “+” OCReq “+” Shared SecretLow16))

where “Address” and “OCReq” are the values of the corresponding parameters in the X-EBO-Opt header with any leading or trailing whitespace characters removed.

#### Enforcement

In order to enforce authentication without also impacting EBO-3 performance, the browser and optimizer shall limit the number of request/response transactions that may occur before authentication has completed.  In other words, EBO-3 allows some unauthenticated optimization leakage, but only a fixed amount.  Specifically, the browser shall not send more than 4 total requests on an EBO connection before it has responded to the optimizer’s challenge.  If the optimizer receives a 5th request without receiving a response to its challenge, the optimizer shall respond to the 5th request with a 401 Unauthorized response and close the connection.  Additional details of this requirement are provided in Appendix A.

# Optional EBO-3 Features

This section describes the features that should be implemented in an EBO-3 browser / optimizer, but are not mandatory.

## Optimization Personalization

This EBO feature allows the user to control how much optimization the optimizer should perform.  The optimizer supports five different levels optimization, 5 being the highest (most compression) and 1 being the lowest (least compression).  This allows the end-user to select the best setting for their handset, network, and browsing preferences.  Without this feature, all users receive the operator default optimization level.  To support this EBO feature, the browser should expose a new UI configuration that allows the user to choose between two or more optimization levels.  For example, the user could pick a value 1-5, or perhaps simply select between “high, medium, and low” speed settings, which could be mapped to 5, 3, and 1, respectively.

Note: In certain deployments, the optimizer may be configured to not honor the user’s preferred optimization level and instead require the optimizer to enforce the default optimization level.  Furthermore, this user setting can of course only be honored when an EBO capable optimizer is deployed in the operator’s network.

## Selective Image Retrieval

This EBO feature allows the browser to selectively disable image optimization for a single request by setting a special EBO request flag that tells the optimizer not to perform any image optimization for that content.  One or both of the following methods should be used to implement this feature:

- Modify the browser’s “reload” functionality, or create a new “reload images” button that sets the special EBO request flag, “Disable Image Compression”, on all re-requested objects.  This would allow the user to visit a page once with the default optimization level, and then press the browser’s reload button to re-download the page’s original images.  This is helpful, for example, if the page included one or more important images that are best viewed without image optimization.

- Modify the browser’s UI to allow the user to select a particular image and then replace only that image with the original version of the image.  Again, the browser would simply need to re-request the image with the special EBO request flag, “Disable Image Compression”, set such that the optimizer would disable image optimization during the second download.  Of course, designing the UI to implement this approach may not be feasible on some devices.

## Image Resizing

The EBO-3 browser should allow the optimizer to perform Image Resizing where possible.  This EBO feature lightens the browser’s CPU load by offloading some of the image resizing work that the browser would otherwise perform to shrink images to the handset’s screen size.  To facilitate this, the browser should communicate the dimensions of its screen size to the optimizer and also indicate whether large images should or should not be resized to fit the screen’s width.  The optimizer shall use this information to automatically resize any images larger than the screen’s width to be equal to the screen’s width.  Since the optimizer uses a standard image resizing algorithm, the resulting images appear the same to the end-user regardless of whether the browser or the optimizer does the resizing.

Note: If the browser actually resizes images to be slightly smaller than the screen’s width, then the browser should advertise the maximum displayable screen size instead of the actual screen size.

Note: If the user has selected a browsing mode in which the browser disables image resizing, the browser shall inform the optimizer of this configuration by disabling the “Enable Image Resizing” setting.

# EBO-3 Header Definitions

This section defines the EBO-3 headers.  There are three EBO-3 request headers:

- X-EBO-UA

- X-EBO-Info

- X-EBO-Req

And there are three EBO-3 response headers:

- X-EBO-Opt

- X-EBO-Resp

- X-EBO-Pred

The optimizer shall remove all of the EBO-3 request headers before forwarding an HTTP request to an origin server or proxy.

## The X-EBO-UA header

The X-EBO-UA header shall have the following comma separated value pair format:

X-EBO-UA: param1=value1, param2=value2, … paramX=valueX

### X-EBO-UA Requirements

- The browser shall include the X-EBO-UA header in the first request on a non-EBO connection to advertise its support for EBO-3.  This header shall always include the BID and BCReq parameters.

- This header shall not include any identifiable user information.

- If the browser does not receive an X-EBO-Opt header in response to its X-EBO-UA header, the browser shall intermittedly include the X-EBO-UA header on new connections to detect when an EBO optimizer becomes available (e.g., on the first new connection of each browsing session).

### X-EBO-UA Parameters

- BID

  - This parameter shall specify the browser’s Browser ID, EBO Generation, and Handset Class as defined in section 5.  The format of the BID parameter shall be “BrowserID.EBOGeneration.HandsetClass”.

  - Example: BID=1.3.0

  - Default Value: none

- BCReq

  - This parameter shall provide the browser’s authentication challenge request.  The value of this parameter shall be a 128-bit hexadecimal number encoded in 32 ASCII bytes.

  - Example: BCReq=0123456789ABCDEF0123456789ABCDEF

  - Default Value: none

An example of a complete X-EBO-UA header is as follows:

X-EBO-UA: BID=1.3.0, BCReq=0123456789ABCDEF0123456789ABCDEF

## The X-EBO-Opt header

The X-EBO-Opt header shall have the following comma separated value pair format:

X-EBO-Opt: param1=value1, param2=value2, … paramX=valueX

### X-EBO-Opt Requirements

- The optimizer shall include the X-EBO-Opt header in the first response on a non-EBO connection.  In this case, the header shall always include the Address and BCResp parameters.

- The optimizer shall include the X-EBO-Opt header in the first response on an EBO connection.  In this case, the header shall always include the Address, BCResp, and OCReq parameters.

- The optimizer’s configurations settings (i.e., Name, Address, and Settings parameters) shall be identical on EBO connections and non-EBO connections.

### X-EBO-Opt Parameters

- Name

  - This parameter shall describe the optimizer (e.g., name and version number) and should be used for informational and debugging purposes only.

  - Example: Name=OSN Monaco 3.2

  - Default Value: none

- Address

  - This parameter shall provide an address in dotted-decimal notation that the browser connects to for all EBO connections.  The browser shall always use this proxy address for EBO connections, and shall not use this proxy address for non-EBO connections.

  - Example: Address=1.2.3.4:80

  - Default Value: none

- BCResp

  - This parameter shall provide the optimizer’s response to the browser’s authentication challenge request.  The value of this parameter shall be a 128-bit hexadecimal number encoded in 32 ASCII bytes.

  - Example: BCResp=0123456789ABCDEF0123456789ABCDEF

  - Default Value: none

- OCReq

  - This parameter shall provide the optimizer’s authentication challenge request to the browser.  The value of this parameter shall be a 128-bit hexadecimal number and encoded in 32 ASCII bytes.

  - Note: This parameter shall be included on EBO connections. This parameter shall not be included in non-EBO connections.

  - Example: OCReq=0123456789ABCDEF0123456789ABCDEF

  - Default Value: none

- Settings

  - This parameter shall be a 32-bit hexadecimal encoded bit-map that advertises the optimizer’s EBO capabilities and settings.  Zero-padding is not required.

  - Example: 1

  - Default Value: 0

  - Bit-map definition:

| Bit Mapping (numbered from lowest order bit) | Setting |
| --- | --- |
| 0 | Stream Compression |
| 1-31 | Reserved |

Figure 6: X-EBO-Opt Settings bit mapping.

  - Stream Compression

    - This is a 1-bit browser setting that specifies whether stream compression is (1) or is not (0) supported.

    - Bit-Mask: 0x00000001

    - Default Value: 0

An example of a complete X-EBO-Opt header on a non-EBO connection is as follows:

X-EBO-Opt: Name=Bytemobile Monaco 3.2, Address=1.2.3.4:80, Settings=1, BCResp=0123456789ABCDEF0123456789ABCDEF

An example of a complete X-EBO-Opt header on an EBO connection is as follows:

X-EBO-Opt: Name=Bytemobile Monaco 3.2, Address=1.2.3.4:80, Settings=1, BCResp=0123456789ABCDEF0123456789ABCDEF, OCReq=0123456789ABCDEF0123456789ABCDEF

## The X-EBO-Info header

The X-EBO-Info header shall have the following comma separated value pair format:

X-EBO-Info: param1=value1, param2=value2, … paramX=valueX

### X-EBO-Info Requirements

- The browser shall include the X-EBO-Info header in the first request on an EBO connection.  In this case, the header shall always include the BID, UID, and BCReq parameters.

- The browser shall include the X-EBO-Info header in the next request after receiving the optimizer’s X-EBO-Opt header.  In this case, the header shall always include the OCResp parameter and shall not include the BID, UID, Proxy, or BCReq parameters.

- This browser may include the X-EBO-Info header in any subsequent request if the Screen or Settings parameters have changed.  In this case, the header shall not include any other parameters.

### X-EBO-Info Parameters

- BID

  - This parameter shall provide the exact same information that the browser provides in the X-EBO-UA BID parameter (i.e., the browser’s Browser ID, EBO Generation, and Handset Class).  The format of the BID parameter shall be “BrowserID.EBOGeneration.HandsetClass”.

  - Example: BID=1.3.0

  - Default Value: none

- UID

  - This parameter shall provide a unique identifier that the optimizer uses for license enforcement.  The format of the UID shall be the same as the standard GUID format, as shown in the example below.

  - Example: UID=00112233-4455-6677-8899-aabbccddeeff

  - Default Value: none

- Proxy

  - This parameter shall allow the browser to provide a proxy address in dotted-decimal notation for the optimizer to use for all outgoing requests, if necessary.  If the browser provides this parameter, the optimizer shall forward all outgoing requests to this proxy address.  Otherwise, the optimizer shall forward requests directly to the content servers.

  - Example: Proxy=5.6.7.8:80

  - Default Value: none

- BCReq

  - This parameter shall provide the browser’s authentication challenge request.  The value of this parameter shall be a 128-bit hexadecimal number and shall be encoded in 32 ASCII bytes.

  - Example: BCReq=0123456789ABCDEF0123456789ABCDEF

  - Default Value: none

- OCResp

  - This parameter shall provide the browser’s response to the optimizer’s authentication challenge request.  The value of this parameter shall be a 128-bit hexadecimal number and shall be encoded in 32 ASCII bytes.

  - Example: OCResp=0123456789ABCDEF0123456789ABCDEF

  - Default Value: none

- Screen

  - This parameter shall specify the device’s screen size (width x height).

  - Example: Screen=480x640

  - Default Value: none

- Settings

  - This parameter shall be a 32-bit hexadecimal encoded bit-map that advertises the browser’s EBO capabilities, settings, and preferences.  Zero-padding is not required.

  - Example: Settings=B

  - Default Value: 0

  - Bit-map definition:

| Bit Mapping (numbered from lowest order bit) | Setting |
| --- | --- |
| 0-2 | Optimization Level |
| 3 | Enable Prediction |
| 4 | Enable Image Resizing |
| 5 | Disable Images |
| 6 | Disable JavaScript |
| 7 | Disable CSS |
| 8 | Disable GIF Animation |
| 9-31 | Reserved |

Figure 7: X-EBO-Info Settings bit mapping.

  - Optimization Level

    - This shall be a 3-bit browser setting that specifies the user’s desired optimization level.  The encoding shall be as follows:

      - 000 – Operator Default

      - 001 – Lowest optimization level (least optimization)

      - 010 – Low optimization level

      - 011 – Medium optimization level

      - 100 – High optimization level

      - 101 – Highest optimization level (most optimization)

      - 110 - Reserved

      - 111 – Reserved

    - Bit-Mask: 0x00000007

    - Default Value: operator defined

  - Enable Prediction

    - This shall be a 1-bit browser setting that specifies whether the browser does (1) or does not (0) support EBO Prediction.

    - Bit-Mask: 0x00000008

    - Default Value: 0

  - Enable Image Resizing

    - This shall be a 1-bit browser setting that specifies whether the browser will (1) or will not (0) resize images larger than the width of the screen, as provided in the X-EBO-Info Screen parameter.

    - Bit-Mask: 0x00000010

    - Default Value: 0

  - Disable Images

    - This shall be a 1-bit browser setting that specifies whether the browser will (0) or will not (1) display images within a page.

    - Bit-Mask: 0x00000020

    - Default Value: 0

  - Disable JavaScript

    - This shall be a 1-bit browser setting that specifies whether the browser will (0) or will not (1) execute internal and external JavaScript code.

    - Bit-Mask: 0x00000040

    - Default Value: 0

  - Disable CSS

    - This shall be a 1-bit browser setting that specifies whether the browser will (0) or will not (1) apply internal and external CSS formats.

    - Bit-Mask: 0x00000080

    - Default Value: 0

  - Disable GIF Animation

    - This shall be a 1-bit browser setting that specifies whether the browser will (0) or will not (1) display GIF animations.

    - Bit-Mask: 0x00000100

    - Default Value: 0

An example of a complete X-EBO-Info header in the first request of an EBO connection is as follows:

X-EBO-Info: BID=1.3.0, UID=00112233-4455-6677-8899-aabbccddeeff, Proxy=5.6.7.8:80, Screen=480x640, Settings=B, BCReq=0123456789ABCDEF0123456789ABCDEF

An example of a complete X-EBO-Info header that would be sent in response to an optimizer’s authentication challenge request is as follows:

X-EBO-Info: OCResp=0123456789ABCDEF0123456789ABCDEF

An example of a complete X-EBO-Info header that would be sent in the event of a configuration setting change is as follows:

X-EBO-Info: Screen=640x480, Settings=A

## The X-EBO-Req header

The X-EBO-Req header shall have the following comma separated value pair format:

X-EBO-Req: param1=value1, param2=value2, … paramX=valueX

### X-EBO-Req Requirements

- The browser shall include the X-EBO-Req header in all requests on EBO connections.  The header shall always include the Key parameter.

### X-EBO-Req Parameters

- Key

  - This parameter shall specify a 32-bit unsigned decimal integer that uniquely identifies each EBO request within a single EBO connection between the browser and the optimizer.

  - Example: Key=3

  - Default Value: none

- Flags

  - This parameter shall be a 32-bit hexadecimal encoded bit-map that specifies the EBO request flags that apply to this EBO request.  Zero-padding is not required.

  - Example: Flags=1

  - Default Value: 0

  - Bit-map definition:

| Bit Mapping (numbered from lowest order bit) | Flag |
| --- | --- |
| 0 | Abort Request |
| 1 | Predicted Request |
| 2 | Disable Image Compression |
| 3-31 | Reserved |

Figure 8: X-EBO-Req Flags bit mapping.

  - Abort Request

    - This shall be a 1-bit response flag that specifies whether the request is (1) or is not (0) an abort message (as defined in section 2.2.2).

    - Bit-Mask: 0x00000001

    - Default Value: 0

  - Predicted Request

    - This shall be a 1-bit response flag that specifies whether the request is (1) or is not (0) a predicted request.  The Predicted Request flag shall only be set if the browser is issuing the request as a result of an X-EBO-Pred URL.

    - Bit-Mask: 0x00000002

    - Default Value: 0

  - Disable Image Compression

    - This shall be a 1-bit response flag that specifies whether the optimizer shall (1) or shall not (0) disable image optimization for this particular request (as described in section 3.2).

    - Bit-Mask: 0x00000004

    - Default Value: 0

## The X-EBO-Resp header

The X-EBO-Resp header shall have the following comma separated value pair format:

X-EBO-Resp: param1=value1, param2=value2, … paramX=valueX

### X-EBO-Resp Requirements

- The optimizer shall include the X-EBO-Resp header in all responses on EBO connections.  The header shall always include the Key parameter.

### X-EBO-Resp Parameters

- Key

  - This parameter shall specify a 32-bit unsigned decimal integer that uniquely identifies the EBO request associated with this EBO response.

  - Example: Key=3

  - Default Value: none

- Flags

  - This parameter shall be a 32-bit hexadecimal encoded bit-map that specifies the EBO response flags that apply to this EBO response.  Zero-padding is not required.

  - Example: Flags=1

  - Default Value: 0

  - Bit-map definition:

| Bit Mapping (numbered from lowest order bit) | Flag |
| --- | --- |
| 0 | Bypass Request |
| 1 | Image Compressed |
| 2 | Image Resized |
| 3 | Image De-animated |
| 4-31 | Reserved |

Figure 9: X-EBO-Resp Flags bit mapping.

  - Bypass Request

    - This shall be a 1-bit response flag that specifies whether the request is (1) or is not (0) bypassed (as described in section 2.2.2).

    - Bit-Mask: 0x00000001

    - Default Value: 0

  - Example: Flags=1

  - The default value of all X-EBO-Resp flags bits including reserved bits shall be 0.

  - Default Value: 0

- Bypass

  - This parameter shall specify an EBO bypass rule (as described in section 2.2.2).

  - Examples:

    - Domain: 	Bypass=abc.com

    - Hostname:	Bypass=www.abc.com

    - Partial URL: 	Bypass=abc.com/foo/bar

    - Full URL: 	Bypass=www.abc.com/foo/bar/index.html

  - Default Value: none

## The X-EBO-Pred header

The X-EBO-Pred header shall have the following comma format:

X-EBO-Pred: url1 url2 url3 … urlN

### X-EBO-Pred Requirements

- The optimizer may include an X-EBO-Pred header in a response on an EBO connection to inform the browser to start requesting certain URLs early.

- Each predicted URL shall be fully qualified, including the protocol (e.g., http://) and port number (if not equal to port 80).  For example,

X-EBO-Pred: http://www.abc.com/xyz/url1.jpg http://www.abc.com:8080/xyz/url2.js

# License Enforcement Parameters

In order to properly license EBO technology, the optimizer tracks the number of unique EBO handsets that have connected to the optimizer in a given time period.  To facilitate this, the browser shall provide four different pieces of information to the optimizer:

- Browser ID

  - The Browser ID uniquely identifies the EBO browser company.  This is used to allow Bytemobile to share EBO revenue collected from the operator with the proper browser company.  This identifier is assigned by Bytemobile and will not be changed in the future.

  - Example: 1

- EBO Generation

  - The EBO Generation reflects the level of EBO supported by the browser.  This document defines EBO Generation 3 and all of the features that are required for EBO-3 compliance.

  - Example: 3

- Handset Class

  - The Handset Class specifies the platform, or device class of the connecting device.  The specifics of how the Handset Class is used are not yet defined.

  - Note: This specification will be updated when these details are defined in the future.  Until that time, the browser shall just advertise Handset Class “0” for all handsets.

  - Example: 0

- User ID

  - The User ID shall uniquely identify a particular handset and allows the optimizer to track the number of unique EBO handsets that have connected in a given time period.  The browser shall generate the User ID with the following requirements

    - The browser shall generate the User ID such that the probability of a collision is very low.

    - The browser shall ensure that “cloned” or otherwise “flashed” handsets generate distinct User IDs, even in the event that the clone occurred after the original handset had already connected to an EBO optimizer.

    - The browser should maintain the User ID across browser upgrades and updates.

  - Example: 00112233-4455-6677-8899-aabbccddeeff

All of this information shall be communicated to the optimizer in the correct formats in order for the browser to be EBO-3 compliant.  The details of this communication exchange are provided in section 4.

# Appendix A: EBO Authentication Scenarios

This appendix describes four EBO Authentication scenarios and the performance implications of each scenario.

Figure 10 shows the most common scenario, in which the browser initially has only one request to send on an EBO connection (e.g., the first request is for an HTML object).  In this case, the challenge response is sent in the 2nd request on the connection and there is no performance degradation, as shown.

Req1: BCReq

Resp1: BCResp, OCReq

Browser

Optimizer

Browser sends only 1 request to start

Req2: OCResp

Second request is sent after first response is received

Authentication is completed in the 2nd request

First response is sent with optimization

Resp2

Resp3

Resp4

Resp5

Figure 10: EBO Authentication – Scenario 1

The next two figures depict a less common case in which the browser initially has 5 or more requests to send immediately on an EBO connection (e.g., the HTML content was cached or was served on a non-EBO connection).  In Figure 11, the browser sends all 5 requests without waiting for the optimizer’s challenge request.  In this case, the optimizer serves the first 4 responses with optimization and then rejects the 5th request by sending a 401 Unauthorized response and closing the connection, as shown.

Req1: BCReq

Req2

Req3

Req4

Req5

Resp1: BCResp, OCReq

Resp2

Resp3

Resp4

Resp5: 401 Unauthorized

Browser

Optimizer

Browser sends 5 requests to start

5th request is rejected for not authenticating

Authentication challenge is received after 5th request was sent

First 4 responses are sent with optimization

Figure 11: EBO Authentication – Scenario 2

In Figure 12, the browser instead sends only the first 4 requests immediately and holds the 5th request.  Once the optimizer’s challenge request is received, the browser releases the 5th request and any additional requests that have been delayed.  This allows the optimizer to complete the authentication process before allowing more than 4 request/response transactions as shown.  As depicted in this figure, there may still be some performance degradation as a result of delaying the 5th request.  However, this delay can be no larger than one round-trip time, and is likely to be negligible in practice.

Req1: BCReq

Req2

Req3

Req4

Resp1: BCResp, OCReq

Resp2

Resp3

Resp4

Browser

Optimizer

Browser sends 4 requests to start

Authentication is completed in the 5th request

Browser delays 5th request until first response is received

First 4 responses are sent with optimization

Req5: OCResp

Resp5

Figure 12: EBO Authentication – Scenario 3

In Figure 13, the browser initially has only 4 requests to send and does not ever generate a 5th request.  In this case, the optimizer eventually closes the idle connection before the authentication process has completed, which means that the mobile operator is not charged for this connection.  However, since only one authenticated connection is required per month in order to collect revenue for a given user, this limitation is not likely to have a significant revenue impact.

Req1: BCReq

Req2

Req3

Req4

Resp1: BCResp, OCReq

Resp2

Resp3

Resp4

Browser

Optimizer

Browser sends 4 requests to start

Optimizer closes the idle connection

Browser receives the challenge request but does not have any more requests to send

First 4 responses are sent with optimization

Idle Connection Close

Figure 13: EBO Authentication – Scenario 4